# Standard Operation Procedure – acidification of slurry/manure

### Name of the analysis:

Lab scale acidification of slurry/manure

### Purpose of the analysis:

To know the amount of acid, which is, needed to reach a specified pH value e.g. before slurry application in a field experiment.

### Method standard:

“Homemade”

Acidification of manure is based on a chemical principle. In the manure there is ammonia (NH3), which is a gas that evaporates, and ammonium (NH4 +), which is bound in the manure in the form of nutrients. There is a chemical equilibrium between NH3 and NH4 +. By lowering the acidity of the manure, the equilibrium is shifted towards ammonium (NH4 +) and virtually no ammonia (NH3) is formed in the manure. When there is no ammonia, it cannot evaporate either.

### Reagents and equipment:

- Analytical scale, watch, pH-meter; magnetic stirrer, magnet, pipette and tip (100 µl) and a 500/1000 ml beaker.

- Slurry/manure.

- 95 % Sulfuric acid.

- Safety goggles, gloves and lab coat. https://dk.vwr.com/assetsvc/asset/da_DK/id/7668377/contents

https://kiros.dk/Web/navigator?action=details&key=22787&lager=&lang=

Work accordingly to the guidelines in www.Kiros.dk  and to the guidelines and safety instructions given by teachers, Laboratory Technologists or assistants in the laboratory.

### Sample preparations:

Make sure it the sample is as homogeny as possible.

### Reagents preparations:

None

### Calibration:

### Calibrate the scale, pipette and the pH-meter according to their manuals.

### Procedure:

- Calibrate the pH-meter according to its manual before start.

- Mix the sample and weigh of 250 gram in a suitable beaker.

- Add a magnet to the beaker and start the magnetic stirrer.

- Measure the pH in the bottom of the beaker to avoid possible foam –note the value.

- Add 100 µl 95% sulfuric acid wait 60 sec. before pH is noted.

- Repeat step 5 until you have reached the desired pH.

- Repeat step 5 one or two times more just to make sure the reached pH is correct.

- Make duplicates.

### Calculations:

### Results:

### Clean up & waste handling:

- Thin slurry can be poured out in the sink using a lot of water. Otherwise, collect it in a suitable bucket for later destruction in Foulum.

- Wash the beaker and magnet.

- Clean the pH-meter

- Wipe the table with a wet clout

### Observations:

- Be aware of cattle and digested biogas slurry create a lot of foam during the acidification.

- If lot of foam occur make sure the pH are measured in the liquid and not in the foam.

- If you add 1000 µl instead of 10x100 µl the pH will be unstable for longer time.

- In cattle manure, the pH will drop immediately after adding acid and then increase during the 60 seconds.

- In pig manure, the pH change will appear after approximate 20 seconds after adding acid and keep decrease until 60 seconds.

- If the content of dry matter are high, you might need additional mixing during the acidification.