# Overview

20 containers with samples, 18 with cattle-slurry, 2 ammonium carbonate standards

9 with packed soil, 9 with soil collected on field experiment day with surface preserved

3 types of treatments, H2SO4, acetic acid and no acid (untreated)

## Sample Naming Convention

Field soils = F

Packed soils = P

H2SO4 treatment = H2SO4

Acetic acid treatment = AA

No treatment = U

Number relates to container/jar

Ie. Sample in container 18 will be described as 18.F.AA

## Experimental Setup

# Day -2 Dry Agro soil

…

# Day -1: Prepare samples, cattle slurry, analysis and setup

Tasks are in prioritized order

Samples: F needs to be collected, P needs to be packed, STD prepare solution

Slurry: Collect and acidify H2SO4 and AA + untreated. Check pH before and after!

Analysis: field-soil water content analysis, slurry TAN analysis, slurry DM analysis, STD ammonia content analysis.

Setup: check flows, prepare acid-trap, test Picarro-cycle, change Any desk code

## Collect cattle slurry and soil-samples, test soil water content

Remember to calibrate the pH meter

Collect cattle slurry

- Stored in building close to L30 (G31?)

- Grap a large bucketful, will be weighed in the lab ≈ 1.5-2 L

Collect soil samples

- Building connected to main Foulum building PV24?

- Cellar

- Cooling room number X?

- Reol 8 kasse 139

- From cattle experiment - 2025-10-28

Test of F-soil water content

- L32

- Weigh 3 aluminum containers, weigh soil from 3 F-samples in container, put into oven.

- Meanwhile, check whether the red lids can be easily removed without spilling soil

- Take out of oven and weigh again after 24 h.

## Slurry acidification:

500 g of slurry per treatment

- Check pH beforehand and afterwards, as triplicates

H2SO4

Using same weight as large-scale will require 0.79 mL 95% wt. H2SO4 (check detailed calculations)

All added at once. Or over time 100 μL (except last addition) each time with 1-minute intervals between addition

AA

100 μL each time, wait for 1 minute and measure pH, until similar to H2SO4 slurry.

Storage

- 100 mL of each for long-term storage

- Rest, use for analysis same day, when done store in closed container in cooler or outside if there is no risk of frost.

## Slurry TAN and DM analysis

As triplicates for each slurry treatment?

- Coordinate with Janni in the interval 11:00-14:00

## Field soil preparation

- Choose 9 samples, mark the sides, remove the top and film them. Leave them for next day - cooler or simply on the lab table clearly marked

## Packed soil preparation

- Collect soil from outside storage at Biogas-plant

- Sieve the soil by removing stones and similar - allows the clumps of soil. Mix afterwards.

- Mark a soil for scratching the surface 2 mm

- 37.5 g soil per packing (150 g in total)

- Add XXX mili-Q water to mimic field conditions

Packing procedure

- Weigh the soil and pack it (use the right stamp, smallest to largest!)

- Scratch the soil evenly

- Add water

- Repeat 4 times

Storage

- Mark and film the samples

- Store in cooler or lab table, clearly marked

## Standard preparation

- 2.27 ammonium carbonate (NH4HCO3) and 20 g potato starch, 100 mL milli-Q water

- Dissolved starch in 60 ℃ sufficient milli-Q water

- At room temperature, add the ammonium carbonate, adjust volume to 100 mL

- Measure exact nitrogen content [gN / L] using ammonium test kit

- Store in cooler.

## Setup preparation

- Start the pump Tune flows to 0.225 L/min

- Set up valve-cycles and test the changes.

- Change Any Desk password to windtunnel

Acid trap:

2.5 L total

0.05 M H2SO4, use approx. 6.85 mL

## Soil analysis

Do only one test, don’t want to sacrifice more soil-samples than necessary

Can also be done the next day if time is short

- 20 g soil

- 20 mL dem. Water

- Shake tube

- Centrifuge for 12 min, 4200 RPM - remember counterweight

- Measure pH of supernatant (liquid fraction)

# Day 1: Start of the experiment

## Setup tests and slurry assessment before starting

- Ensure all sample-flows are 0.225 L/min, before experimental start

- Ensure valve-shifts work as intended

- Note time-delay between picaro and actual time

- Measure pH of slurries as triplicates

## Start the experiment, prepare current sample

Approx 2-3 min before valve shift, remove plastic-wrap from current sample

Add 2 g of needed slurry as drops, in small as droplets

Exchange red plastic lid, with petri-dish

Add to Picarro setup

Note picaro-time for the shift, and the flow of the sample stream, and bypass

# Day: 2-6: Extract data via Any desk

Download yesterday’s raw Picarro file into own folder via Any Desk

Repeat for next day

# Day 7: End of the experiment

Note Picarro-time for the end of the experiment.

Download raw Picarro file for the final day.

Note bypass and sample flows, is expected to have a slight decrease - an average between start and end will be used.

Remove the samples from the setup, can most likely dilute and put into sink.

# Calculation details

## Slurry acidification:

Amount of H2SO4

0.18 kgacid60.05·10-3 tslurry=2.99·kgacidtslurry

If using 500 g in lab:

mH2SO4= 0.500 kgslurry·10-3tslurrykgslurry·2.99kgacidtslurry=0.001495·kgacid·103=1.495 g

ρH2SO4 95 %wt=1.8837gmL

V=mρ=1.495 g1.8837gmL=0.79·mL

## Packed soils

Amount of soil to use:

Bulk density of soil during the field experiment: 1.50gcm3±0.09gcm3

Vsample=100 cm3

msoil,  sample=100 cm31.50gcm3±0.09gcm3=150 g±9 g

msoil,  pack=150 g±9 g4=37.5±2.25

Amount of water to add

Field gravimetric water content:

Ufield, cattle, dry=mwatermsoil= 0.17gwatergsoil

Agro soil has 15.5 % water content calculated on a “wet” basis

U agro, wet=mwatermsoil+mwater=0.155

Uagro,dry=0.1551-0.155=0.183

Conversion - redefine mass of water:

Uwetmsoil+mwater=mwater

Uwetmsoil+Uwatermwater=mwater

Uwetmsoil=mwater-Uwetmwater

Uwetmsoil=mwater(1-Uwet)

Uwetmsoil(1-Uwet)

Insert into Dry definition

Udry=mwatermsoil=Uwatermsoil1-Uwatermsoil=Uwater(1-Uwater)

## Setup - Acid trap

C1V1=C2V2

C2=0.05 M

V2=Vwater+V1=2.5 L

If using is 95 %Wt. H2SO4:

ρsolution=1.88337 g mL-1

For 1000 mL

msolution=1883.37 g

mH2SO4=1883 g·0.95=1789.20 g

nH2SO4 =1789.20 g98.079 g mol-1=18.24molL=C1

V1=C2V2C1=0.05 M·2.5 L18.24 M·103mLL=6.85307·mL

Vwater=2500 mL-6.85307·mL=2493.15·mL